Proxmox server issue

[Company name] | [Company address]

Avril 2026

sebastien trudel

2026

Table of Contents

The problem:	2

The fix:	4

Conclusion:	6

Other potential fixes:	7

References:	7

After a power outage at the hosting location (3ti), the server wouldn’t boot.

Connected remotely to a computer located at the hosting provider, connected to IPMI/iDRAC then used virtual console to see what was being displayed on the server.

# The problem:

Error was:

--------------------------------------------

Time out for waiting the udev queue being empty.

Time out for waiting the udev queue being empty.

Gave up waiting for root file system device. Common problems:

- Boot args (cat /proc/cmdline)

  - Check rootdelay= (did the system wait long enough?)

- Missing modules (cat /proc/modules; ls /dev)

ALERT! /dev/mapper/pve-root does not exist. Dropping to a shell!

BusyBox v1.35.0 (Debian 1:1.13.0-4+b7) built-in shell (ash)

Enter ‘help’ for a list of built-in commands.

(initramfs)_

--------------------------------------------

This built-in shell lacks required tools to perform basic diagnostics let alone any rescue operations, but ls /dev wouldn’t list the hard drives.

Checked in the DRAC/Perc/BIOS and the drives, RAID and virtual disks are all healthy so there is probably no hardware failure.

A reboot shows that GRUB does load so the drives ARE working.

A search for “/dev/mapper/pve-root does not exist proxmox” shows this is a common issue and there are a few ways people have fixed them.

- Use a different kernel. When updating the kernel Proxmox keeps older kernel versions and they can be accessed by selecting the “Advanced options for Proxmox Virtual Environment GNU/Linux” boot option in GRUB.

  - https://www.reddit.com/r/homelab/comments/1cetljo/i_need_help_devmapperpveroot_does_not_exist/

- Use Chroot to perform a rescue operation on the Linux installation. Including installing a new kernel if the Proxmox doesn’t have other installed kernels.

  - https://forum.proxmox.com/threads/alert-dev-mapper-pve-root-does-not-exist.148029/

- Add “rootdelay=10” to GRUB

  - https://forum.proxmox.com/threads/solved-i-need-help-dev-mapper-pve-root-does-not-exist.116967/

- Adding “GRUB_CMDLINE_LINUX_DEFAULT="intel_iommu=on iommu=pt"” or “quiet intel_iommu=off intremap=off“ to GRUB

I tried option 3 by adding it directly in the GRUB selection menu to no avail, it didn’t change anything. I assumed option 4 didn’t have anything to do with my issue and wasn’t even worth a try.

Option 1 did not help either. I couldn’t see an older kernel in the advanced option. Either the system didn’t update or it didn’t keep the previous kernel version for me to boot on.

I then went on to try and boot from an Ubuntu desktop live USB drive. But even from there I couldn’t see the hard drive listed in /dev, or in lsblk or lshw, etc. At least I now had access to better tools that initramfs was giving me that I could use to perform a better diagnosis.

- dmesg -T | tail -n 200 (look for PCIe, SAS, SATA, NVMe, link resets)

- lsblk -e7 -o NAME,TYPE,SIZE,MODEL,SERIAL,TRAN,HCTL (see what the kernel created)

- lspci -nn | egrep -i 'sas|raid|sata|nvme|scsi' (confirm the controller exists)

As I was getting desperate, I was basically trying anything I could find only, I eventually saw “megaraid_sas 0000:01:00.0” in “journalctl -k -b|egrep -i ‘mpt3sas|megaraid|ahci|nvme|reset|timout|aer’.

The RAID card I use is a Dell PERC h310 so where is that “megaraid” device coming from? It also has log entries saying “reset/timeout”, bingo, I thought, this is the issue, it. times out??

Now lspci does find something if I filter for megaraid (or lspci -k -s 01:00.0) instead of trying to find “Dell” or “PERC”.

There were also some messages in “dmesg -T | egrep -I ‘ahci|ata|SATA’”: SATA (1 to 5) link down (SStatus 0 SControl 300). I assumed this was related to the timeout with the megaraid card as it is a SAS/SATA compatible RAID device and my drives are all SATA.

I tried changing the drives to RAID in the server BIOS configuration even though it didn’t make sense, and anyway no changes were made prior to the server stopping working. The fact that I could get to GRUB meant the system sees the disks, at least prior to GRUB or the OS being loaded. Not seeing the disks even in the Ubuntu live I took it that it meant that it was probably a type of driver issue, not a hardware problem. I began looking for a different PERC card that might be using a different driver like an H710 hoping that the new card would be seen properly by the OS and that the RAID configuration would transfer over to the new card.

# The fix:

From chroot I added the Proxmox to the deb list and installed kernel 6.14

To chroot:

Need an OS that can see the disks then mount the folders to the temporary OS

- mount --rbind /proc /mnt/proc

- mount --rbind /sys /mnt/sys

- mount --rbind /dev /mnt/dev

- mount --rbind /run /mnt/run

Then chroot to the mounted broken install: “chroot /mnt” then “mount -a” to mount other partitions that might be needed.

Once in chroot, I checked which kernel I could try, the official documentation says 6.14 is available as an option.

Using this procedure I installed the 6.14 kernel by adding an entry to the repository list.

sudo nano /etc/apt/sources.list.d/pve-install-repo.list

deb [arch=amd64] http://download.proxmox.com/debian/pve bookworm pve-no-subscription

sudo apt update

sudo apt install proxmox-kernel-6.14

The server boots fine after installing the 6.14 kernel. The comments said you could force a default kernel for Proxmox to boot to with the “Proxmox-boot-tool” kernel pin command.

Running “proxmox-boot-tool kernel list” gives the following results:

--------------------------------------------

Manually selected kernels:

None.

Automatically selected kernels:

6.14.11-8-bpo12-pve

6.8.12-23-pve

--------------------------------------------

Running “proxmox-boot-tool kernel pin 6.14.11-8-bpo12-pve” outputs:

--------------------------------------------

E: no kernel image found in /boot for '6.14.11-8-bpo12-pve', not setting default.

Possible Proxmox kernel versions are:

6.14.11-8-bpo12-pve

6.8.12-23-pve

--------------------------------------------

Assuming the “Automatically selected kernels” from kernel list means the new kernels are already “pinned”.

“uname -r” gives 6.8.12-23-pve which is good enough for me for now. I would like to make sure the kernel updates aren’t done without approval though, so I still need to figure out how to do that.

In the meantime, I ran these commands which hopefully works well to stop the kernel from updating.

- apt-mark hold proxmox-kernel-6.14.11-8-bpo12-pve-signed

- apt-mark hold proxmox-kernel-6.14

- apt-mark hold proxmox-kernel-6.8.12-23-pve-signed

- apt-mark hold proxmox-kernel-6.8.12-9-pve-signed

- apt-mark hold proxmox-kernel-6.8

--------------------------------------------

root@proxmox3:~# apt list --installed |grep kernel

WARNING: apt does not have a stable CLI interface. Use with caution in scripts.

proxmox-default-kernel/stable,now 1.1.0 all [installed]

proxmox-kernel-6.14.11-8-bpo12-pve-signed/stable,now 6.14.11-8~bpo12+1 amd64 [installed,automatic]

proxmox-kernel-6.14/stable,now 6.14.11-8~bpo12+1 all [installed,upgradable to: 6.14.11-9~bpo12+1]

proxmox-kernel-6.8.12-23-pve-signed/stable,now 6.8.12-23 amd64 [installed,automatic]

proxmox-kernel-6.8.12-9-pve-signed/stable,now 6.8.12-9 amd64 [installed]

proxmox-kernel-6.8/stable,now 6.8.12-23 all [installed,upgradable to: 6.8.12-25]

proxmox-kernel-helper/stable,now 8.1.4 all [installed]

root@proxmox3:~# apt-mark hold proxmox-kernel-6.14.11-8-bpo12-pve-signed

proxmox-kernel-6.14.11-8-bpo12-pve-signed set on hold.

root@proxmox3:~# apt-mark hold proxmox-kernel-6.14

proxmox-kernel-6.14 set on hold.

root@proxmox3:~# apt-mark hold proxmox-kernel-6.8.12-23-pve-signed

proxmox-kernel-6.8.12-23-pve-signed set on hold.

root@proxmox3:~# apt-mark hold proxmox-kernel-6.8.12-9-pve-signed

proxmox-kernel-6.8.12-9-pve-signed set on hold.

root@proxmox3:~# apt-mark hold proxmox-kernel-6.8

proxmox-kernel-6.8 set on hold.

--------------------------------------------

The bug seems to have been identified in this Proxmox forum thread: https://forum.proxmox.com/threads/pve-8-2-kernel-6-8-4-2-does-not-boot-cannot-find-root-device.145764/

By pschneider1968 (Peter Schneider) who submitted it to Proxmox Bugzilla (and to the Linux mailing list?) with the author of the offending patch (member of the kernel SCSI team) in CC. They then spent time troubleshooting the issue on Peter’s machine

The offending code is at the bottom of this commit I believe: https://git.kernel.org/pub/scm/linux/kernel/git/stable/linux.git/commit/?id=6d20acbf3e3a32d331947dbc3802cf2d1a399e7d

--------------------------------------------

drivers/scsi/scsi.c | 7 +++++++

1 file changed, 7 insertions(+)

diff --git a/drivers/scsi/scsi.c b/drivers/scsi/scsi.c

index 3e0c0381277a..f0464db3f9de 100644

--- a/drivers/scsi/scsi.c

+++ b/drivers/scsi/scsi.c

@@ -350,6 +350,13 @@ static int scsi_get_vpd_size(struct scsi_device *sdev, u8 page)

if (result < SCSI_VPD_HEADER_SIZE)

return 0;

+		if (result > sizeof(vpd)) {

+			dev_warn_once(&sdev->sdev_gendev,

+				      "%s: long VPD page 0 length: %d bytes\n",

+				      __func__, result);

+			result = sizeof(vpd);

+		}

+

result -= SCSI_VPD_HEADER_SIZE;

if (!memchr(&vpd[SCSI_VPD_HEADER_SIZE], page, result))

return 0;

--

2.44.0

--------------------------------------------

# Conclusion:

I believe that an update to the kernel probably caused the issue. Reading online seems to indicate that this is somewhat common with older cards like mine. I did not have good backups in place yet, they were automatically done but on the same RAID as the read of the data, which is of course bad practice, but I was planning on sending these to my other server automatically eventually.

For now, I have an external USB drive connected where the VM backups are stored.

# Other potential fixes:

https://forum.proxmox.com/threads/timed-out-for-waiting-for-udev-queue-being-empty.129481/

- Chroot then modify the thin_check_options in /etc/lvm/lvm.conf near line 1120

# References:

Proxmox boot tool how tos: https://dannyda.com/2023/11/07/proxmox-ve-pve-boot-tool-kernel-tools-how-to-change-kernel-manually-pin-kernel/

Other threads with similar issues:

- https://forum.proxmox.com/threads/q-weird-problem-after-upgrade-prox7-8-boot-newer-kernel-alert-dev-mapper-pve-root-does-not-exist-not-bootable.164002/

General Linux help:

- https://cr0x.net/en/proxmox-disks-not-detected-checklist/

- https://oneuptime.com/blog/post/2026-03-02-how-to-check-hardware-information-with-lshw-dmidecode-and-lspci-on-ubuntu/view

- How to chroot https://support.scc.suse.com/s/kb/How-to-Chroot-in-Rescue-Mode?language=en_US

- How to chroot: https://www.turnkeylinux.org/docs/chroot-to-repair-system

- Manually pin kernel https://dannyda.com/2023/11/07/proxmox-ve-pve-boot-tool-kernel-tools-how-to-change-kernel-manually-pin-kernel/